Signal Strength Lab

Name: _____________________Class: _____________   Date: _________

Subject 							Ms. _____________________

Signal Strength Lab:

Problem

How can you represent the relationship between an electromagnetic signal and distance/location?

Hypothesis

What do you think will happen?

Write your hypothesis in the “If…(independent variable), then…(dependent variable)” format?

IV-Distance

DV-Signal strength

If _________________________________________________________________

then _________________________________________________________________.

Materials

- Transmission devices:

  - Baby monitors, electronic car key, radio control cars, wireless microphone or walkie talkies.

- COSMOS collection Database

- SDR receiver (500 hz -1.7 Ghz)

- Measuring tool (ribbons cut into 1.5 m or 15 meters long)

- Laptops

- Post-it Chart Paper

- Chair to hold the COSMOS toolkit in place

- Markers

- Graph Paper

- Graphic Organizer to record data

- Vocabulary Sheet for the students

Procedure

Pre-lab preparations (Must be done prior to part A and part B)

- In your group, decide which job each person will have

  - Walkie-Talkie 1 Signal Transmitter: _______________________________________________________

  - Walkie-Talkie 2 /Signal Receiver: _______________________________________________________

  - Timer: _________________________________________________

  - Data Recorder/Program Runner: ______________________________________________________

  - Facilitator/Overseer of the whole Experiment

- Set up COSMOS data collection program.

  - Make sure that you have two walkie-talkies per group.

  - Each group will be assigned at different frequencies.

  - Make sure that both persons holding the walkie-talkie are on the same channel  at your designated group.

  - Check if the SDR receiver is attached to your computer and to your antenna.

  - Download the 1-4-walkie-talkie- constellation.grc to have the program running in your computer.

  - Test if the program is running and make sure your group should be link to a specific radio frequency different from other groups.

  - Your group should consist only of 5 people.

  - Each person in the group will have to take the responsibility of the task assigned to them.

  - The overseer is in-charge of making sure that everyone in the group is on task and that the whole experiment is done correctly.

  - You’re all responsible to take care of  the COSMOS Toolkit.

- Discuss how you will orchestrate the whole process or create a system where you can perform  the whole process within the allotted time.

- Wait for your teacher’s instructions before starting the experiment.

Part A- Outside (example in a schoolyard, park or open field)

- Each group can choose a spot on the playground/park  far from other  teams, where you wanted to perform the experiment.

- Put your COSMOS toolkit on a flat surface/top of the chair.

- Stand at the starting point, this will be your zero meter.

- Timer should start the clock as the transmitter starts moving. This experiment should be done in 30 minutes.

- The transmitter should travel  along a straight path, moving 5 meters away (about 16-17 feet) for every stop.

- The transmitter push the button of the walkie-talkie for every stop and communicates with the receiver on the walkie-talkie  to record the data.

- The recorder looks at the screen and write down the signal strength seen from the computer screen.

- Repeat steps 9-11 until the the transmitter reaches 50 meters.

Part B-Indoors (Receiver  in classroom, transmitter in hallway)

DOOR CLOSED:

- Complete steps 1-4 in “Pre-lab Preparations”

- All receivers for each team stay in the classroom and choose a specific corner/spot in the room to set up the COSMOS toolkit in place.

- Wait for your teacher to give you permission to go into the hallway if you are the transmitters.

- Stand at the starting point, this will be your zero meter.

- Timer should start the clock as the transmitter starts moving. This experiment should be done in 30 minutes.

- The transmitter should travel  along a straight path (if the hallway is a 50 meter stretch)  moving 5 meters away (about 16-17 feet) for every stop. Otherwise, indicate in your data collection the shape of your school’s hallway (example U-shape, an L -shape, etc.) coz this will show different results of the experiment.

- The transmitter push the button of the walkie-talkie for every stop and communicates with the receiver on the walkie-talkie  to record the data.

- The recorder looks at the screen and write down the signal strength seen from the computer screen.

- Repeat steps 6-8 until the the transmitter reaches 50 meters.

DOOR OPEN:

- Find out the results of the same experiment with the door open in another period/session.

- Repeat steps 1-8

Data Collection Sheets:

Part A-Signal Strength Outdoors

| Distance in Meters | Signal Strength in Db |
| --- | --- |
| 0 |  |
| 5 |  |
| 10 |  |
| 15 |  |
| 20 |  |
| 25 |  |
| 30 |  |
| 35 |  |
| 40 |  |
| 45 |  |
| 50 |  |

Part B-Signal Strength Indoors

| Distance in Meters | Signal Strength in Db |
| --- | --- |
| 0 |  |
| 5 |  |
| 10 |  |
| 15 |  |
| 20 |  |
| 25 |  |
| 30 |  |
| 35 |  |
| 40 |  |
| 45 |  |
| 50 |  |

Guide Questions:

- What have you noticed from the data gathered in A? How is the signal strength affected as the transmitter moves away every 5 meters?

- Compute for the percentage of change in the signal (whether it’s an increase or decrease) as the transmitter moves away every 5 meters.

- Create a table and transform each percentage change in decimal and fractional form.

| Distance (in meters) | Signal Strength | Percentage (Decrease or Increase) | Fractional (Decrease or Increase) | Decimal (Decrease or Increase) |
| --- | --- | --- | --- | --- |
| 0-5 | -16.92 dB to -17.47dB |  |  |  |
| 5-10 |  |  |  |  |
| 10-15 |  |  |  |  |
| 15-20 |  |  |  |  |
| 20-25 |  |  |  |  |
| 25-30 |  |  |  |  |
| 30-35 |  |  |  |  |
| 35-40 |  |  |  |  |
| 40-45 |  |  |  |  |
| 45-50 |  |  |  |  |

- What have you noticed from the data gathered in B? How is the signal strength affected as the transmitter moves away every 5 meters?

- Repeat  steps 2-3.

- Did you notice any significant difference? Why or why not?

- How does your data look like if you plot these points on a Cartesian Plane?

ANALYSIS/CONCLUSIONS:

Using the data that you collected in part A and part B of your experiment, create a  graph to plot the points from your own table and make some observations.

Remember to give your graph a title, and to label the X-axis and the Y-axis.

Part A-Graph

Questions:

- What do you notice about the data you collected?

__________________________________________________________________________________________________________________________

- Based on you graphical data, what can you conclude about the relationship between distance and signal strength? 
_______________________________________________________________________________________________________________________________________________________________________________________

Part B-Graph

- What do you notice about the data you collected in B?

__________________________________________________________________________________________________________________________

- Based on you graphical data, what can you conclude about the relationship between distance and signal strength? 
_______________________________________________________________________________________________________________________________________________________________________________________

Organize your data on a chart paper/graphing chart paper.

Ask your teacher for a designated spot to post your results on the wall of your classroom.

Prepare your post-its and pens for your group-gallery walk.

Using your post-its, ask questions or give a comment.

Each group will rotate in a counter-clockwise direction staying in a poster for 3 minutes each. The bell rings to indicate the time to switch or to rotate.

CLASS DISCUSSION:

Each group will hold their poster and look at all the comments and questions.

You will be given 5-7  minutes to discuss how to address or incorporate  the questions/comments of other people gathered from the post-its during class discussions.

Group presentations